### 1. Simple Analogies Se Samjho

#### A. Bicycle Kickstand (Over-Center Mechanism)

Cycle ke stand ko dhyaan se dekho:

- Stand ya toh poora UP (Retracted) rehta hai ya poora DOWN (Parked).

- Jab aap pair se stand ko aage/peeche dhakka dete ho, ek 'Center Point' aata hai.

- Us Center Point ko cross karte hi, spring stand ko apne aap 'KHAT' se uski final position par kheinch leti hai.

- Parkbnb mein bhi same geometry rehti hai: Center point cross hote hi barrier apne aap DOWN ya UP position par set ho jata hai.

#### B. Car Door Latch (Locking Principle)

Car ka door kholte waqt motor darwaza nahi dhakelti, sirf latch (taala) kholti hai:

- Door band rehta hai hook/latch ki wajah se.

- Handle ya motor lock ko 0.5 second ke liye release karti hai.

- Door thoda sa pop-out hota hai, aur baaki hinges door ko movement dete hain.

### 2. Spring Ka Asli Roll Kya Hai?

Clock Spring (ya Torsion Spring) basically ek Energy Storage Device hai (jaise measuring tape ke andar hoti hai).

- Jab Barrier UP hota hai: Spring pull (stretch) position mein hoti hai aur energy stored rehti hai.

- Jab Lock Khulta hai: Spring stored energy ko release karti hai aur barrier ko initial push deti hai.

- Constant Force / Clock Spring kyun?: Kyunki ye bahut kam jagah (compact) leti hai, sasti hoti hai, aur 360-degree rotation easily handle karti hai.

### 3. Step-by-Step System Flow

[ BARRIER UP ]  -->  [ APP UNLOCK ]  -->  [ LATCH RELEASE ]  -->  [ GRAVITY + SPRING PUSH ]  -->  [ BARRIER DOWN ]

(Locked position)     (0.5 sec Motor)     (Hook khula)          (Over-Center Cross)             (Stable Flat)

- POSITION 1: Barrier UP (Locked)

  - Barrier seedha khada hai.

  - Lock Hook barrier ko hold karke rakhta hai.

  - Spring compressed/tensioned hai.

- STEP 2: Unlock Command (App)

  - Chhoti si motor ya Solenoid Actuator sirf 0.5 second ke liye chalegi.

  - Motor barrier ko nahi girati; ye sirf Hook/Latch ko 5 mm peeche khinchtii hai.

- STEP 3: Downward Movement

  - Latch khulte hi spring ki stored energy barrier ko 5–10 degree aage dhakelti hai.

  - Geometry Over-Center cross karti hai.

  - Gravity aur Spring force barrier ko smooth way mein ground par DOWN gira deti hai.

- STEP 4: Barrier DOWN (Stable)

  - Barrier zameen par flat hai.

  - Motor aur Spring par 0% load hai.

  - Car aaram se barrier ke upar se nikal kar park ho sakti hai.

- STEP 5: Return to UP Position

  - Car ke nikalne ke baad, return spring ki mechanical force barrier ko wapas UP pull karti hai.

  - Top par pahunchte hi Latch par CLICK aawaz aati hai aur mechanical lock engaged ho jata hai.

### 4. Is Design Ke 3 Sabse Bade Fayde

- Zero Gearbox & Tiny Motor: Heavy motor aur expensive gearbox ki zaroorat khatam. Ek tiny 12V servo motor ya solenoid hi kafi hai kyunki usse koi weight nahi uthana.

- Extreme Battery Life: Motor har cycle mein sirf 0.5 second power consumption karti hai. Battery saal bhar chal sakti hai.

- 100% Structural Protection: Car agar barrier ke upar chadh bhi jaye, toh force direct Steel Top Plate -> Steel Base Plate -> Concrete Ground par transfer hota hai. Motor, PCB, aur Lock 100% safe rehte hain.

What would you like to do next?

Draft mechanical CAD sketch requirements

Create state machine diagram for PRD

List local manufacturing vendors in Indore

# Motion & Locking Mechanism Concept (V1)

## Design Philosophy

The barrier movement is mechanically assisted, while the actuator is used only for locking and unlocking. This minimizes power consumption, reduces component count, and improves long-term reliability.

## Key Components

- Rigid U-shaped Barrier (Steel Tube + Rubber Cover)

- Dual Pivot Shaft

- Dual Torsion Springs (Left & Right) – Motion Assist Only

- Single Steel Rotary/Hook Lock

- Mini Solenoid (Lock Release)

- Hall Position Sensor

- Reinforced Steel Chassis & Base Plate

## Design Principles

- Motor/Solenoid performs only lock/unlock operation.

- Torsion springs assist barrier movement; they never carry structural loads.

- Vehicle impact loads are transferred through:
Barrier → Pivot → Chassis → Base Plate → Ground

- Lock bears the holding load, not the spring or actuator.

- Dual torsion springs distribute forces evenly across both pivot shafts.

## Operating Flow

### 1. Locked State

Barrier UP

│

Steel Lock Engaged

│

Spring Preloaded

│

System Secure

### 2. Unlock

App Command

│

Mini Solenoid Pulse

│

Steel Lock Released

│

Barrier Free

### 3. Opening

Barrier Released

│

Torsion Springs Assist

+

Gravity

│

Barrier Moves DOWN

### 4. Open State

Barrier DOWN

│

Vehicle Passes / Parks

│

System Waits

### 5. Closing

App Close Command

│

Return Mechanism Assists

│

Barrier Moves UP

│

Steel Lock Auto Engages

│

System Secure

## Design Advantages

- Low power consumption (actuator operates only during lock/unlock)

- No heavy drive motor or gearbox required

- Reduced mechanical complexity

- Lower BOM cost

- Easy field maintenance

- High impact resistance

- Suitable for local fabrication using laser-cut steel, welded chassis, and standard mechanical components

## Future Research Items

- Final return mechanism (manual assist, cable assist, or compact linear actuator)

- Lock geometry optimization

- Spring stiffness selection

- Over-center motion optimization

- Prototype testing and fatigue validation

Note: The exact return mechanism and spring characteristics will be finalized after prototype testing and CAD validation. The overall architecture is designed to keep structural loads away from the spring and actuator while maintaining a simple, low-cost, and robust mechanical system.