# Parkbnb Bike

## Smart Two-Wheeler Parking Lock

### Product Requirements Document (PRD)

# 1. Product Overview

Parkbnb Bike is a smart, automatic front-wheel locking parking system designed for motorcycles and scooters. Unlike traditional parking locks that block the vehicle, Parkbnb securely captures the front wheel using a mechanical locking mechanism, enabling app-controlled access, reservation, and anti-theft protection.

# 2. Design Philosophy

Mechanical First • Low Power • High Reliability

- Vehicle enters naturally.

- Wheel automatically locks.

- Motor performs only unlock operation.

- Mechanical components carry all structural loads.

- No continuous motor operation.

# 3. Product Objectives

- Automatic wheel locking

- App-based unlocking

- Low power consumption

- Anti-theft protection

- Easy manufacturing

- Modular construction

- Suitable for outdoor environments

# 4. Mechanical Architecture

Motorcycle

Front Wheel

│

Wheel Guide Funnel

│

Self-Centering Cradle

│

Steel Wheel Lock Hook

│

Mini Solenoid Release

│

Steel Structural Frame

│

Base Plate + Anchors

# 5. Key Components

### Mechanical

- Front Wheel Guide

- Self-Centering Wheel Cradle

- Steel Lock Hook

- Spring Loaded Lock

- Mini Solenoid

- Dual Guide Rollers

- Steel Chassis

- Base Plate

### Electronics

- ESP32

- Hall Sensor

- Lock Sensor

- Battery

- Bluetooth / WiFi

- Optional 4G

# 6. Working Principle

## Step 1

User reserves parking.

↓

App unlocks stand.

↓

Wheel cradle opens.

## Step 2

Bike enters.

Wheel

↓

Guide Funnel

↓

Wheel Cradle

Guide automatically centers wheel.

## Step 3

Wheel reaches locking position.

↓

Spring-loaded steel hook rotates.

↓

Lock automatically engages.

↓

Parking confirmed.

## Step 4

System Status

Wheel Present

↓

Lock Engaged

↓

Secure

## Step 5

Leaving

User presses Unlock.

↓

Mini Solenoid releases hook.

↓

Bike reverses.

↓

Spring resets lock.

↓

Ready for next user.

# 7. Safety Features

- Mechanical auto-lock

- Manual emergency release

- Wheel presence verification

- Lock engagement verification

- Tamper detection

- Battery monitoring

- Event logging

# 8. Sensors

### Standard

- Hall Sensor

- Lock Position Sensor

### Optional

- Ultrasonic

- Camera

- IMU

# 9. Camera Features (Optional)

- Parking proof image

- Wheel alignment verification

- Tamper snapshot

- Parking occupancy

- Remote live image

# 10. Electronics

- ESP32

- Li-ion Battery

- Solar Charging (Optional)

- BLE

- WiFi

- 4G (Optional)

# 11. Mobile Application

Features

- Reserve Parking

- Unlock Bike

- Lock Status

- Battery Status

- Parking History

- Notifications

- OTA Updates

# 12. Structural Design

Structure consists of:

- Steel chassis

- Wheel guide

- Wheel cradle

- Lock module

- Electronics enclosure

Vehicle load path

Wheel

↓

Cradle

↓

Steel Chassis

↓

Base Plate

↓

Concrete

No structural load reaches electronics.

# 13. Mounting

- Four M10/M12 Anchor Bolts

- Anti-theft fasteners

- Steel mounting plate

- Waterproof sealing

# 14. Power System

Configurations

- Battery

- Battery + Solar

- Battery + AC Adapter

Battery Life Target

- 6–12 months (typical usage)

# 15. Serviceability

Field replaceable

- Lock Module

- Battery

- Solenoid

- PCB

- Sensors

Average service time

<15 minutes

# 16. Target Performance

| Parameter | Target |
| --- | --- |
| Lock Time | <1 second |
| Unlock Time | <1 second |
| Battery Life | 6–12 Months |
| Operating Life | >100,000 Cycles |
| IP Rating | IP67 Target |
| Maintenance | Minimal |

# 17. Manufacturing Strategy

### Buy Components

- Solenoid

- Springs

- Sensors

- Battery

- ESP32

- Fasteners

### Fabricated Components

- Steel Chassis

- Wheel Cradle

- Guide Funnel

- Base Plate

- Lock Bracket

# 18. Future Roadmap

### V2

- Camera AI

- Number Plate Recognition

- Dynamic Pricing

- Smart Reservation

- Fleet Parking

### V3

- Automatic Wheel Position Detection

- Solar Charging

- Digital Key

- Predictive Maintenance

# 19. Unique Selling Proposition (USP)

- Automatic Front-Wheel Locking: The front wheel is automatically centered and mechanically locked without requiring chains or external locks.

- Low-Power Design: Electrical power is used only for unlocking; the mechanical lock secures the bike without continuous power.

- Modular Architecture: Common electronics platform with field-replaceable lock, sensor, and power modules.

- High Security: Steel locking mechanism, tamper detection, and app-controlled access.

- Scalable Platform: The same electronics and software platform can be adapted for two-wheelers, four-wheelers, and commercial parking systems.

## Product Vision

Parkbnb Bike is designed as a smart wheel-docking system, not just a parking stand. By combining automatic mechanical wheel capture, electronic access control, and a modular hardware platform, it aims to provide a secure, low-maintenance, and scalable parking solution for apartments, offices, campuses, metro stations, and public parking facilities. This same platform can later evolve into the four-wheeler version while reusing most of the electronics and software architecture.