# Parkbnb V1

## Low-Cost Mechanical Architecture & Manufacturing Strategy

# Objective

# Develop a low-cost, low-power, robust smart parking lock using a mechanically assisted barrier instead of a continuously motor-driven mechanism.

The design focuses on:

- Minimum BOM Cost (Target: ₹2,000–3,000 Mechanical)

- Easy Manufacturing in Indore

- Low Maintenance

- High Reliability

- Modular Construction

- Standard Off-the-Shelf Components

# Design Philosophy

Unlike conventional parking locks where the motor performs the complete barrier movement, Parkbnb V1 uses mechanical energy for movement while the actuator performs only locking and unlocking.

Design Rule

Motor/Solenoid → Lock & Unlock Only
Springs → Motion Assist Only
Steel Structure → Carries All Vehicle Loads

# Mechanical Architecture

Rubber Protected U Barrier

│

Steel Internal Spine

│

Dual Pivot Shaft

│

Dual Torsion Springs

│

Steel Rotary / Hook Lock

│

Mini Solenoid

│

Steel Chassis

│

Steel Base Plate

│

Concrete Foundation

# Vehicle Load Path

Vehicle

│

Barrier

│

Steel Pivot

│

Steel Chassis

│

Base Plate

│

Anchor Bolts

│

Ground

Important

Vehicle impact shall never be transferred to:

- Springs

- Solenoid

- Electronics

- Battery

# Standard Components (Buy)

| Component | Remarks |
| --- | --- |
| Torsion Springs | Standard Industrial Spring |
| Trailer/Rotary Hook Lock | Heavy Duty Steel |
| Push Solenoid | 12V |
| Hall Sensors | A3144 |
| Magnets | Neodymium |
| Nylon Bushes | Standard Size |
| Fasteners | Standard Hardware |

# Custom Manufactured Parts

Only these parts need fabrication.

Component

Steel Chassis

Base Plate

Side Plates

Pivot Shaft

U Barrier

Lock Bracket

# Manufacturing Process

### Laser Cutting

- Base Plate

- Side Plates

- Lock Brackets

### CNC / Lathe

- Pivot Shaft

- Bush Housing

- Spacer

### Welding

- Steel Chassis

- Barrier Frame

### Powder Coating

Outdoor corrosion protection

### Final Assembly

- Springs

- Lock

- Solenoid

- Sensors

- Electronics

# Estimated Mechanical BOM

| Component | Approx. Cost |
| --- | --- |
| Barrier | ₹400 |
| Shaft | ₹200 |
| Springs | ₹300 |
| Bushes | ₹100 |
| Steel Lock | ₹350 |
| Solenoid | ₹300 |
| Steel Chassis | ₹600 |
| Base Plate | ₹350 |
| Fasteners | ₹100 |
| Hall Sensor | ₹50 |

### Mechanical Cost

₹2,300–2,800

# Existing Products Used as Inspiration

Instead of developing every component from scratch, the design leverages proven industrial products.

| Product | Used For |
| --- | --- |
| Motorcycle Center Stand | Pivot & Spring Mechanism |
| Trailer Over-Center Latch | Locking System |
| Industrial Torsion Spring | Motion Assist |
| Nylon Bush | Pivot Support |
| Steel Floor Enclosure | Base Structure |
| Gate Spring Hinge | Return Mechanism Reference |

# Advantages

- Low BOM Cost

- Low Power Consumption

- No Heavy Gear Motor

- Easy Local Manufacturing

- Modular Design

- Easy Maintenance

- High Structural Strength

- Replaceable Components

# Future Improvements

- Over-Center Linkage Optimization

- Rotary Latch Integration

- Compact Return Mechanism

- Camera-Based Parking Verification

- Ultrasonic Parking Position Detection

- Solar Charging Module

- Predictive Maintenance Dashboard

- OTA Firmware Updates

# V1 Target Specifications

| Parameter | Target |
| --- | --- |
| Mechanical BOM | ₹2,000–3,000 |
| Barrier Cycle Time | <5 sec |
| Power Consumption | Very Low (Lock/Unlock Only) |
| Operating Life | >100,000 Cycles |
| Maintenance | Modular & Field Replaceable |
| Manufacturing | Indore/Pithampur Fabrication |

## Core USP

A mechanically assisted smart parking lock where electrical power is used only for locking and unlocking, while barrier movement is achieved through optimized mechanical design.

This architecture reduces cost, extends battery life, simplifies maintenance, and is well suited for scalable manufacturing using locally available fabrication capabilities.