SMART PARKING LOCK

Product Requirements Document — Hardware Implementation Scope

Sections 1–7  |  v4

# 1. Product Dimensions & Packaging

Defines the physical footprint and packaging requirements for safe transport, storage, and installation across residential, commercial, and government deployments.

## Key Requirements

- Compact footprint compatible with standard parking-bay widths

- Low profile when lowered (open) to minimize tripping / vehicle-clearance risk

- Manageable weight for 1–2 person handling and installation

- Standardized, protective packaging for logistics and storage

## Design Requirements

- Final L × W × H to be confirmed after CAD

- Packaging to withstand transit shock, vibration, and moisture

- Clear handling / orientation labeling on packaging

## Validation

- Dimensional / tolerance check

- Drop test

- Transit vibration test

Dimensions: 46 × 46 × 9 cm; Weight: ~8 kg-(-aprox acc. Parkobot)

# 2. Structural Design

Defines the mechanical architecture of the product — modular, durable, and serviceable across residential, commercial, and government deployments.

## Key Modules

- Barrier Module — vehicle blocking, impact absorption, branding

- Drive Module — motor, gearbox, coupling, pivot shaft

- Chassis Module — structural rigidity, load distribution

- Housing Module — environmental & mechanical protection

- Mounting Module — anchoring and ground interface

## Construction

- Barrier: UV-resistant rubber/PU cover over a segmented aluminium or glass-filled nylon spine, with impact-absorbing foam and an internal steel safety cable

- Chassis: 2–3 mm powder-coated galvanized steel with dedicated mounts for motor, battery, lock, and PCB

- Housing: sealed, UV-stabilised enclosure with replaceable gasket (IP67 target)

- Pivot: stainless-steel shaft with replaceable bushes and mechanical end stops

## Design Targets

- Design life ≥ 5 years  |  ≥ 100,000 operating cycles  |  Safety factor ≥ 2

- Corrosion & UV resistant, IP67 target, no exposed sharp edges or pinch points

# 3. Locking Mechanism

Core design rule: the motor moves the barrier; a separate steel lock secures it. This separation keeps holding loads off the motor and gearbox.

## Proposed Components

- 12V DC geared motor

- Spring-loaded, hardened-steel locking pin (deadbolt)

- Solenoid or micro-servo for pin release

- Two Hall sensors (raised / lowered position) + one lock-engagement sensor

- Motor-current sensor for jam / forced-movement detection

- Concealed manual key release for power-failure access

## Operating Sequence

Authorized command → lock disengages → sensor confirms → motor moves barrier → position sensor confirms → mechanical lock engages → secure status shown.

## Tamper / Overload Response

- Forced movement is absorbed by the steel pin and chassis, not the motor

- Buzzer, LED, and app/event-log notification trigger on unexpected movement

- On heavy impact, the flexible arch yields in a controlled manner while lock, motor, and electronics stay protected

- Lock remains mechanically engaged during power loss; no continuous power required

Terminology note: raised barrier = Locked/Protected Position; lowered barrier = Open Position. Keep this convention consistent across CAD, firmware, and testing.

self-locking worm gear-

# 4. Motion System

Delivers smooth, controlled, and repeatable barrier motion, distinct from the locking/security function covered in Section 3.

## Key Requirements

- Smooth start/stop motion with no abrupt jerks

- Defined, repeatable open/close cycle time

- Real-time obstruction / overload detection during motion

- Stable performance across temperature and load variation

## Components

- DC geared motor

- Motor driver / controller

- Transmission (gearbox / coupling)

- Position feedback sensors (Hall / encoder)

## Design Requirements

- Cycle time target: ≤5 seconds open/close (to be validated)

- Auto-stop and reverse on obstruction detection

- Manual motion override during power failure

## Validation

- Cycle time test

- Repeatability test

- Obstruction detection test

# 5. Mounting System

A secure, vandal-resistant mounting system enabling fast installation and long-term stability on concrete, asphalt, and paver surfaces.

## Key Components

- Reinforced steel base plate + chemical/expansion anchor bolts

- Mounting bracket with anti-tamper fasteners

- Rubber sealing gasket and alignment template

## Design Requirements

- Minimum 4 mounting points; hidden/protected, corrosion-resistant fasteners

- Anti-pry base geometry; waterproof seal between housing and ground

Design principle: the mounting system must be stronger than the barrier assembly, so external forces reach the ground without loosening the device.

# 6. Battery & Power Housing

A safe, weather-resistant, serviceable compartment for the battery and power components, supporting multiple charging options.

## Power Configurations

- Battery only  |  Battery + Solar  |  Battery + AC Adapter

## Key Requirements

- IP67-ready sealed compartment with secure mounts for battery and BMS

- Ventilation for heat dissipation; waterproof cable entry points

- Modular battery tray for quick, tool-assisted replacement

- Battery physically isolated from the motor and gearbox

(Battery life: Up to 4–6 months (typical use))--parkobot

# 7. Serviceability

The product must allow quick inspection, repair, and part replacement in the field — without full disassembly — backed by a predictive, modular, remote-first maintenance strategy that keeps lifetime ownership cost low.

## Maintenance Levels

| Level | Description |
| --- | --- |
| L1 | Cleaning & Inspection |
| L2 | Battery & Barrier Replacement |
| L3 | Motor, Lock & PCB Replacement |
| L4 | Factory Repair (Major Damage) |

## Field-Replaceable Components & Typical Service Time

| Component | Replace Time |
| --- | --- |
| Battery Pack | 5 min |
| Barrier Module | 10 min |
| Motor & Gearbox | 15 min |
| Main PCB | 10 min |
| Communication Module | 5 min |
| Sensors | 5 min |

- Also field-serviceable: charging module, seals & gaskets

## Warranty

- 1-year standard warranty covering manufacturing defects

- Free OTA firmware updates and free mobile app updates

## Predictive Maintenance (Key USP)

- Continuous monitoring: battery health, motor current, lock status, sensor status, communication status, tamper events, temperature

- On issue detection: alert to owner + dashboard, service ticket auto-generated

## AMC Plans

| Plan | Residential | Commercial | Government |
| --- | --- | --- | --- |
| Basic | Free (1-yr warranty) | Included | Included |
| Standard | ₹**/year | /unit/year | Custom SLA |
| Premium | ₹,***/year | /unit/year | Custom SLA + 24×7 support |

## Remote Maintenance

- OTA firmware updates, device diagnostics, configuration changes, log collection, health monitoring, remote restart — reduces technician visits

## Preventive Maintenance Schedule

| Interval | Activity |
| --- | --- |
| Every 6 months | Cleaning, bolt inspection, functional test |
| Every 12 months | Battery health check, seal inspection, firmware update |
| Every 3–5 years | Battery replacement (usage-dependent) |

## Service Dashboard

- Device health, battery %, last online, signal strength, motor & lock status, error codes

- Service history, warranty status, AMC status

## Design Targets

- Average annual maintenance cost: ₹200–500 per unit

- Average service time: <20 minutes

- Remote issue resolution: >80%

- Device uptime: >99%

USP: Predictive maintenance, remote diagnostics, OTA updates, and modular components — designed to minimize downtime and reduce lifetime ownership cost.

# Conclusion — What This Document Covers

This document defines the complete Hardware Implementation Scope for the Smart Parking Lock across seven core areas:

- Product Dimensions & Packaging — footprint, weight, and transit/packaging requirements

- Structural Design — mechanical architecture, modules, materials, and design targets

- Locking Mechanism — motor + steel-lock separation, components, and tamper response

- Motion System — drive components, cycle time, and obstruction handling

- Mounting System — installation, anchoring, and validation requirements

- Battery & Power Housing — power configurations and housing requirements

- Serviceability — maintenance levels, modular replacement, warranty, predictive maintenance, AMC plans, remote maintenance, and service targets

Electronics Implementation Scope, Project Timeline, and Research Items are covered separately and will be added in a follow-up version.